Using ExpertPRIZ environment

Exercise nr. 3

Developing a program package

- Develop a program package ”Warmwall”, which calculates the actual heat resistance of the building elements (walls and ceilings). Compare found heat resistance value with  established norms in Estonia. Calculation rules are given here in a simplified form.

- Calculation of heat resistance

Roi	- boundary layer thermal resistance, i=1 … n

Ro	- the total thermal resistance

Di	- boundary layer thickness mm

li	- heat conductivity of the material W/m2*C

Roi	= di/(li*1000)

Ro	= Roi

A decision about heat resistance

Boundary total heat resistance compared with normal resistance –

Ro>3 W/m2*C – walls

Ro>4 W/m2*C – ceilings

If the condition is met, you can survive the winter in this building.

- Building material data:

Name				Heat conductivity of the material W/m2*C

cyproc				0,81

daub				0,21

particleboard			0,23

gaskukermit			0,24

reinforced concrete		2.04

sandlime			1,1

tep-plate			0.15

glass wool (Ahlström)		0.045

styrofoam 			0,045

rubberoid			0,10

bitumen coating		0,15

glassine			0,18

- Example

  - Main knowledge base

wmexpert.exp

?		numleyers	?	>		result

!clear y	1		?r1	r1<3		not OK, too cold

!file warmcalc	1		?r1	r1>=3		okey

2		?r2	r2<3		not OK, too cold

2		?r2	r2>=3		okey

  - Textfile ‘warmcalc’ with Solveri input language text 

r1=k1thickness/(l1*1000)

k1thickness=100

ex1 expert ex_base=’mater.exp’ result=l1

r2=r1+k2thickness/(l2*1000)

k2thickness=100

ex2 expert ex_base=’mater.exp’ result=l2

  - Expert system of material choosing ‘mater.exp’

mater.exp

materjal	result

cyproc		0,81

…