Student Name: _______________________   Student Code:  __________   Variant: ___

BIST Optimization

Choose circuits from Table 1 and constraints from Table 2 according to your variant.

Example: variant 5  circuits: C1908, C3540; C1908 constraint – M; C3540 constraint – T

- Pseudo-random test generation:

- Find out the maximum fault coverage for given circuits applying tuned ATPG.

- Choose “good” seed and polynomial to generate effective pseudo-random test.

- Generated pseudo-random test sequences must have the maximum fault coverage and the length of test must be in the range T<= length<=1,2*T where T – Time Constraint from Table 2. (Example: c1908  7500 <= test length <= 9000).

- Use Type I and Type II LFSR generators.

- Reseeding algorithm:

- Find complete test (max. fault coverage) for given circuit, using the best PRPG (Type I LFSR) test sequence achieved in task 1.

- Perform experiments stepping through the constraint values and choose 5 results (test must comply with the constraints specified in your variant). Example: c1908 M = 1, 2, 3…15 (min. step = 1); T = 7500, 7000, 6500… (min. step = at least 5% of T).

- Hybrid BIST algorithm:

- Find complete test (max. fault coverage) for given circuit, using the best PRPG (Type I LFSR) test sequence achieved in task 1.

- Perform experiments and choose 5 evenly distributed results (test must comply with the constraints specified in your variant).

- Results evaluation:

- According to the marginal results obtained in task 2 and 3 construct the Cost curves on the same graph for Reseeding and Hybrid BIST algorithms.

- Compare results of Reseeding and Hybrid algorithms.

Table 1

| Variant: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Circuits: | C432 | C499 | C1355 | C880 | C1908 | C3540 | S1196 | S1269 | S1423 | S832 |
|  | S1196 | S1423 | S832 | S1269 | C3540 | C880 | C499 | C1908 | C432 | C1355 |

Table 2

| Circuits | Time Constraint (patterns): (T) | Memory Constraint (patterns): (M) | α | Cost (C) Constraint C=T+ α*M | Reseeding |  | Hybrid BIST |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Variant – odd | Variant – even | Variant – odd | Variant – even |
| C432 | 550 | 10 | 40 | 550 | M | T | M | T |
| C499 | 2200 | 10 | 70 | 2200 | M | T | M | T |
| C1355 | 2000 | 10 | 50 | 2000 | M | T | M | T |
| C880 | 10000 | 10 | 600 | 10000 | M | T | M | T |
| C1908 | 7500 | 15 | 400 | 7500 | M | T | M | T |
| C3540 | 16000 | 30 | 400 | 16000 | T | M | T | M |
| S1196 | 30000 | 20 | 1500 | 30000 | T | M | T | M |
| S1269 | 1000 | 10 | 70 | 1000 | T | M | T | M |
| S1423 | 30000 | 12 | 2500 | 30000 | T | M | T | M |
| S832 | 18000 | 20 | 700 | 18000 | T | M | T | M |

T – number of pseudo-random test patterns(test length). M – number of deterministic patterns(memory vectors).

- Results:

| Circuit: | Type I LFSR |  | Type II LFSR |  |
| --- | --- | --- | --- | --- |
|  | Test Length | Fault Coverage | Test Length | Fault Coverage |

Circuit 1: __________

| Reseeding |  |  |  | Hybrid BIST |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated Cost: | Test Length | Memory Vectors | Fault Coverage | Calculated Cost | Test Length | Memory Vectors | Fault Coverage |
| 1. |  |  |  | 1. |  |  |  |
| 2. |  |  |  | 2. |  |  |  |
| 3. |  |  |  | 3. |  |  |  |
| 4. |  |  |  | 4. |  |  |  |
| 5. |  |  |  | 5. |  |  |  |

Circuit 2: __________

| Reseeding |  |  |  | Hybrid BIST |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated Cost: | Test Length | Memory Vectors | Fault Coverage | Calculated Cost | Test Length | Memory Vectors | Fault Coverage |
| 1. |  |  |  | 1. |  |  |  |
| 2. |  |  |  | 2. |  |  |  |
| 3. |  |  |  | 3. |  |  |  |
| 4. |  |  |  | 4. |  |  |  |
| 5. |  |  |  | 5. |  |  |  |

Test Length – number of pseudo-random test patterns.  Memory Vectors – number of deterministic patterns.

Present BIST Cost curves on the plot below:

Cost

Cost

Circuit 1

Circuit 2

X-axis – Test Length or Memory Vectors (according to your variant)