### Mikrokontrolleri ülesanne

Juhendi leiab siit.
Labori failid leiab siit.

Microcontroller task IAX0583-IAX0584-IAX0590

You can find the manual siit.
Lab files can be found siit.

File:MKKT.pdf - ATI public wiki (ttu.ee)

Microcontrollers Homework

Perform the following task on the development board:

With the switches, 8 bits are entered in one row. The user can enter a selected number of lines, must be used

matrix with dynamically allocated memory. Once the user has entered the data, he can choose whether

an AND/OR/XOR function is performed on each column and the column results are displayed on the LED lights.

The number of rows in use should be displayed on the SSD (seven-segment-display). In addition, make a row

delete function.

Assessment (10p):

Data entry with dynamic memory allocation. (2p)

Row delete function. The row number should be selected from the switches. All the following lines must

move up by one after deletion (an empty hole must not remain). (2p)

Correct answers in logic operations. The logic operation should be selected with the buttons. (2p)

Displaying answers. Also includes display of switch status on LED lights at the input. (2p)

To get maximum points, the development board must always display information to the user on the screen, which

must be done next and display an error text in case of wrong entries. (2p)

To use dynamic memory, it is necessary to increase the size of the heap used in the project by 100000 to a byte

Mikrokontrollerite kodutöö

Tehke arendusplaadil järgmine ülesanne:

Lülititega sisestatakse ühte ritta 8 bitti. Kasutaja saab sisestada valitud arvu ridu, tuleb kasutada

dünaamiliselt eraldatud mäluga maatriksit. Kui kasutaja on andmed sisestanud, saab ta valida, kas

igas veerus teostatakse JA/VÕI/LÕPP-VÕI funktsioon ja veeru tulemused kuvatakse LED-tuledel.

Kasutusel olevate ridade arv peaks olema kuvatud SSD-l (seitsmesegmendilisel ekraanil). Lisaks tehke rea kustutusfunktsioon.

Hindamine (10p):

Andmete sisestamine dünaamilise mälujaotusega. (2p)

Rea kustutamise funktsioon. Rea number tuleks valida lülitite hulgast. Kõik järgmised read peavad

pärast kustutamist ühe võrra ülespoole liikuma (tühja auku ei tohi jääda). (2p)

Õiged vastused loogikatehetes. Loogikatehe tuleks valida nuppudega. (2p)

Vastusi kuvades. Sisaldab ka lüliti oleku kuvamist sisendi LED-tuledel. (2p)

Maksimaalsete punktide saamiseks peab arenduslaud kasutajale alati ekraanil teavet kuvama, mida tuleb teha järgmisena, ja valede sisestuste korral kuvama veateate. (2p)